# Software requirements for diffraction (DREAM, MAGIC, BEER, HEIMDAL)

# Instrument control

|  | DREAM |  |  |
| --- | --- | --- | --- |
| M = Must, S = Should, N = Nice | M | S | N |
| Data Display |  |  |  |
| Full detector area: (n-dimensional (nD): 2θ, d, Q, E, Intensity projected in flexible and “clever” ways for detector testing and calibration, background assessment, environment, shielding | X |  |  |
| 1D live histogram display: Integrated intensity over all detectors vs ToF,d,Q) | X |  |  |
| Save detector and 1D histogram images | X |  |  |
| Region of interest (ROI) selection and readout of physical location of detector module | X |  |  |
| Remote access to live detector data for users |  | X |  |
| Remote access to live detector data for instrument team | X |  |  |
| Incident beam spectrum (bunker): reading from monitor | X |  |  |
| Incident beam spectrum (cave): reading from monitor | X |  |  |
| Correction of the live detector data for distortions, efficiency, calibration, |  | X |  |
| Plot of SEE parameters (field, temp) as a function of time | X |  |  |
| Plot of polarizer parameters (TBD) | X |  |  |
| Choppers speed & phase to have a clear indication for low, medium or high resolutions (simple visualization) | X |  |  |
| Under the Hood |  |  |  |
| All relevant EPICS information saved in NeXuS file | X |  |  |
| sample, user, experiment information in NeXuS file | X |  |  |
| Logbook of entire experiment including instrument configuration, beam status and SEE parameters | X |  |  |
| CLI & GUI Interfaces |  |  |  |
| Driving motors, setting limits, offsets | X |  |  |
| Changing SEE parameters | X |  |  |
| Reading SEE parameters | X |  |  |
| Ability to script counting for time, monitor counts, proton charge | X |  |  |
| Continuous measurements during sweeping of field, temperature, current |  | X |  |
| Loops, if-then | X |  |  |
| Script simulation |  | X |  |
| Quick change of instrument setups (SEE, high/low resolutions, polarized/unpolarized neutrons) | X |  |  |
| Instrument Live feedback |  |  |  |
| Vacuum system (choppers, guide, sample vessel) | X |  |  |
| Beam power monitor | X |  |  |
| Polarizer | X |  |  |
| Video camera inside the cave | X |  |  |
| Shutter position | X |  |  |
| PSS status of cave (locked, swept, opened) | X |  |  |
| Status of removable platform | X |  |  |