Target Wheel Drive and Shaft- Load case definition for pressure and temperature transients

|  | Name | Role/Title |
| --- | --- | --- |
| Owner | <<Name>> | <<Role/ Title>> |
| Reviewer | <<Name>> | <<Role/ Title>> |
| Approver | <<Name>> | <<Role/ Title>> |

| Table of content | Page |
| --- | --- |

1.	Scope	3

2.	Issuing organisation	3

3.	INTRODUCTION	3

4.	Loads during normal operation	3

4.1.	Pressure and temperature	3

4.2.	Mass	3

4.3.	Accelerator	4

5.	operational limits related to accidents or failure of components	8

5.1.	General	8

5.2.	Pressure and temperature	9

5.3.	Rotation	9

5.4.	Proton beam	10

5.5.	Earthquake	10

5.6.	Hydrogen fire	10

5.7.	Drop loads	10

5.8.	Self- induced vibration	10

6.	Operational limits	11

6.1.	Normal operation, full helium blower capacity	11

6.2.	Operation with half helium blower capacity and half beam current	17

6.3.	Operation at 500 MeV	18

7.	references	19

Document Revision history	19

# Scope

This report describes relevant load cases and defines operational limits associated with different operational and accidental situations for the Target Wheel, Drive and Shaft, denoted system 1000.

# Issuing organisation

Target Division, Work Package 2, Target Systems.

# INTRODUCTION

As part of Target structural verification, operational and accidental loads determine load cases which Target Wheel, Drive and Shaft needs to be qualified for. This document aims to describe these loads and define operational limits. In[2] these loads are classified according to the code selected for design, RCC-MRx.

# Loads during normal operation

## Pressure and temperature

| Target inlet pressure | Target inlet temperature | Target mass flow |
| --- | --- | --- |
| 1,1 MPa | 40oC | 3 kg/s |

Table 1. Normal operating thermal conditions

## Mass

| System part | Mass |
| --- | --- |
| Spallation material | 3100 kg |
| Cassette (mass per cassette including spallation material) | 110 kg |
| Vessel up to and including flange towards shaft excluding spallation material and cassette | 3500 kg |
| Shaft including inner- and outer shielding | 10 000 kg |
| Drive unit including X/Y table | 10 000 kg |

Table 2. Masses

## Accelerator

Operation at full beam power (2GeV)

| Load | Name | Description |
| --- | --- | --- |
| 1 | Normal accelerator heat load | The normal operating beam footprint is defined according to [6] and [9]. Uncertainities in rastering and beam power has been accounted for by introducing safety margins. Values to use are presented in Chapter 6. Accelerator macropulse has a duration of 2,86 ms and 14 Hz frequency. Design calculations shall be based on heat deposition based on design beam footprint defined in [6] with accelerator power 5,2 MW [9]. |

Table 3. Accelerator load

Figure 1. Rastering pattern (left) and resulting nominal beam footprint (right). [xx]

Figure 2. Rastering pattern.  In picture above, horizontal rastering frequency is 39,55 kHz, vertical rastering frequency 29,05 kHz. Maximum deflection is 63 mm horizontal and 18 mm vertical. Beamlet RMS size is 15,9 mm horizontal and 5,3 mm vertical.

| Load | Name | Description |
| --- | --- | --- |
| 2 | Temperature transients associated with normal stop and start of accelerator | Frequent start and stop of accelerator, with Helium cooling operational, leading to temperature fluctuation in Target. See below, figure 3 and table 1. The load is described in reference [5]. |

Table 4. Transient load

Figure 3. Graphical presentation of postulated beam trips, from reference[5]

| Table 1 Number of occurrences per event |  |
| --- | --- |
| Event | Occurrences |
| Δσ1 | 1 |
| Δσ2 | 175 |
| Δσ3 | 8981 |
| Δσ4 | 4.36e7 |
| Δσ5 | 39069 |
| Δσ6 | 31219 |
| Δσ7 | 26047 |
| Δσ8 | 21735 |

| Load | Name | Description |
| --- | --- | --- |
| 3 | Local unbalance in helium flow | Steady-state operation with locally increased temperature due to anticipated postulated damage to spallation material. The load is described in reference [10]. One channel between tungsten blocks in the most stressed position is completely blocked. |

Table 5. Local blocking

| Load | Name | Description |
| --- | --- | --- |
| 4 | Operation with one Helium blower | Steady-state operation at 50 % flow with 50 % beam current. The load is the result of failure of one helium blower. It is treated as a separate operating mode with separate operational limits. Operational limits are presented in chapter 6. |

Table 6. Operation with one blower

# Loads related to incidents or failure of components

## General

This group of events are defined based on their mitigation limit. Many events will be mitigated both by the Machine Protection System and by the Target Safety System.

Generally, Machine Protection System is designed to protect the facility from damage, while the Target Safety System is designed to protect both people employed within the ESS as well as the general public from radiological hazard.

Therefore, an error exceeding the limits defined to activate the Target Protection system (Machine Protection System) will occur one or several times during the operation of the ESS. This means that the failure will occur more often than 1 per 100 year, ie the frequency is  >10-2 / year.

Limits for triggering the Target Safety System have been chosen with the sole purpose of protecting from damage that potentially can lead to radiological release. Therefore there may be local high temperature or local permanent deformation associated with events mitigated in this way. A possible outcome of a trip of the Target safety system could be that parts of structure or indeed the whole target will be damaged.

Assigning of relevant RCC- MRx criteria for each load is done in [2].

## Pressure and temperature

| Load | Name | Description |
| --- | --- | --- |
| 5 | Pressure variations | Steady-state operation with reduced operating pressure at undisturbed volumetric flow. The load is related to failure of Pressure Control System. It is described in detail in [4]. |
| 6 | Temperature variations | Steady-state operation at undisturbed pressure and volumetric flow but with varying inlet helium temperature. The load is related to failure in heat exchangers or control of these. It is described in detail in [4] |
| 7 | Flow variations | Steady-state operation at undisturbed pressure and temperature but with varying coolant massflow. This load is based on calculations of either internal or external leakage from cooling system, see [1] |

Table 7. Pressure and temperature transients

## Residual heat

| Load | Name | Description |
| --- | --- | --- |
| 8 | Simultaneous trip of accelerator and helium cooling system | Stop of accelerator and helium blower. Temperature development associated with spallation material residual heat. The load is described in [11]. Calculation of residual heat is presented in[12]. |

Table 8. Residual heat

## Rotation

| Load | Name | Description |
| --- | --- | --- |
| 9 | Rotational speed variations. | Steady- state operation with reduced or increased rotational speed. Operational limits are presented in chapter 6. |
| 10 | Unsyncronized Target rotation | Steady-state operation with varying degree of syncronization with accelerator. Operational limits are presented in chapter 6. |

Table 9. Rotation

## Proton beam

| Load | Name | Description |
| --- | --- | --- |
| 11 | Failure of rastering magnets | Rastering magnets fail leading to unrastered beamlet on target. The load is described in [7] |
| 12 | Failure of quadropoles | Quadropoles fail leading to collapse of beamlet, but with rastering still operational. The load is described in [7]. |

Table 10. Accelerator

## Earthquake

| Load | Name | Description |
| --- | --- | --- |
| 13 | Safe Shutdown Earthquake | Global vibrations from postulated earthquake. Target rotor movement is limited through contact with pedestal. The load is described in reference [13]. |

Table 11. Earthquake

## Rotor unbalance and vibration

| Load | Name | Description |
| --- | --- | --- |
| 14 | Self- induced vibration | Target rotor vibration due to unbalance excitation or excitation from motor (‘Torque ripple’). Postulated excitation frequencies are defined in chapter 6. |

Table 12. Rotor unbalance

# Operational limits

## Background to selected operational limits.

Limits for low massflow and pressure for machine protection system are defined as values which lead to a Tungsten maximum temperature above 500oC, see [14].

Temperature limitations for machine protection system are defined based on temperature limitations for lubricants and bearing clearance.

Limits for massflow and temperature for Target safety system are based on values leading to Tungsten temperature exceeding 700 oC and are defined in [3].

Limits for the accelerator – target interface are defined according to the table below.

For phase deviation between target and accelerator, machine protection system trip limit is based on protecting the uncooled rib structure from impact of the beam footprint encompassing 99 % of beam power. Phase deviation value will be averaged over one revolution.

Low speed limit, for the machine protection system is based on the requirement that Tungsten should only be hit by proton beam once per revolution. The speed limitation is intended to be fast and stop accelerator before next pulse.

For the target safety system, low speed limit is defined by the requirement that not more than 5 pulses should hit the same tungsten during one revolution [6]. High speed limit is based on risk of failure due to resonance. The wheel is stopped at a speed 20 % below first estimated resonance speed.

| Limits for phase deviation |  |  |
| --- | --- | --- |
|  | Normal operation | Machine protection system trip limit |
| Nominal distance between ribs | 208 mm |  |
| Beam footprint drift during pulse | 9 mm |  |
| Accelerator footprint positioning [8] | 10 mm |  |
| Runout affecting beam footprint [15] | 2 mm |  |
| Tolerance in manufacturing of Beam Entrance window [15] | 2 mm |  |
| Sum of errors | 23 mm |  |
| Width of beam footprint encompassing 99,9 % of beam energy [8] | 180 mm |  |
| Width of beam footprint encompassing 99 % of beam energy [8] |  | 160 mm |
| Remaining space available for motion control and syncronization | 5 mm | 25 mm |
| Allocated space for motion control and syncronisation | 4 mm (+/- 2 mm) | 20 mm (+/- 10 mm) |

Table 13. Motivation for selected limits- phase

| Limits for speed |  |  |  |
| --- | --- | --- | --- |
|  | Normal operation | Machine protection system trip limit | Target safety system trip limit |
| Rib thickness | 10 mm |  |  |
| Remaining space left with nominal footprint position | 5 mm | 25 mm | - |
| Total allowable drift before footprints overlap | 15 mm (99,9 % footprint) | 35 mm (99% footprint) | - |
| Allocated allowable drift | 1,5 rpm | 3,5 rpm | - |
| Nominal speed | 23,3333 rpm |  |  |
| Minimum allowable speed | 22 rpm | 20 rpm | 5 rpm |
| Maximum allowable speed | 25 rpm | 27 rpm | 72 rpm |

Table 14- Motivation for selected limits- speed

## Summary of selected operational limits

| Pressure, temperature, flow |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Entity | Nominal value | Warning limit | Warning limit maximum delay | Machine protection system trip limit | Machine protection trip maximum delay | Target safety system trip limit | Target safety system trip limit maximum delay | Passive limiting device such as safety valve limit |
| High helium pressure | 1,1 MPa | 1,15 MPa | 1 s | 1,2 MPa | 5 s | - | - | 1,3 MPa |
| Low helium pressure | 1,1 MPa | 1,05 MPa | 1 s | 1,0 MPa | 5 s | 0,62 MPa | 10 s | - |
| High helium massflow | 3 kg/s | 3,15 kg/s | 1 s | 3,3 kg/s | 5 s | - | - | - |
| Low helium massflow | 3 kg/s | 2,8 kg/s | 1 s | 2,5 kg/s | 5 s | 2 kg/s | 10 s | - |
| High helium temperature | 40oC | 50oC | 1 s | 60oC | 5 s | 240 oC | 10 s | - |
| Low helium temperature | 40oC | 20oC | 1 s | 10oC | 5 s | - | - | - |

Table 15-Summary of pressure and temperature limits

| Rotation |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Entity | Nominal value | Warning limit | Warning limit maximum delay | Machine protection system trip limit | Machine protection trip maximum delay | Target safety system trip limit | Target safety system trip limit maximum delay | Passive limiting device such as safety valve limit |
| Phase error Defined as (mm) on 2616 mm diameter | 0 | 2 mm | Averaged over 1 revolution | 10 mm | Averaged over 1 revolution | - | - | - |
| Low speed | 23,33333 rpm | 22,0 rpm | 0,07s | 20 rpm | 0,07 s | 5 rpm | 0,5s | - |
| High speed | 23,33333 rpm | 25 rpm | 0,07s | 27 rpm | 0,07s | 72 rpm | 0,5s | - |

Table 16- Summary of rotation limits

| Accelerator [8] |  |  |  |  |
| --- | --- | --- | --- | --- |
| Entity | Nominal value | Design value | Rastering failure | Quadropole failure |
| Averaged beam power | 5,0 MW | 5,2 MW | 5,2 MW | 5,2 MW |
| Beam current density | 56uA/cm2 | 71uA/cm2 | 71uA/cm2 | 71uA/cm2 |
| Beamlet size (xy) | 68 mm2 | 50 mm2 | 50 mm2 | 0,25 mm2 |
| 99,9 % Beam fraction footprint size | 180 x 64 mm2 | 180 x 64 mm2 | 68 mm2 | 119x40 mm2 |
| 99 % Beam fraction footprint size | 160 x 60 mm2 | 160 x 60 mm2 | 68 mm2 | 119x40 mm2 |
| Vertical misalignment | 0 mm | 3 mm | 0 mm | 0 mm |
| Horizontal misalignment | 0 mm | 5 mm | 0 mm | 0 mm |

Table 17- Summary of accelerator limits

| Rotor unbalance The table summarizes frequencies which may not coincide with rotor natural frequencies. |  |  |
| --- | --- | --- |
| Nominal frequency | Two times nominal frequency | Nominal frequency times number of pole pairs |
| 0,3-0,5 Hz | 0,6-1,0 Hz | 13 Hz-20 Hz |

Table 18- Summary of frequencies which should not coincide with rotor natural frequencies at normal operation.

## Operation with half helium blower capacity and half beam current

Only values changed with respect to nominal operation are specified.

| Entity | Nominal value | Warning limit | Warning limit maximum delay | Machine protection system trip limit | Machine protection trip maximum delay | Target safety system trip limit | Target safety system trip limit maximum delay | Passive limiting device such as safety valve limit |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| High helium massflow | 1,5 kg/s | 1,6 kg/s | 1 s | 1,8 kg/s | 5 s | - | - | - |
| Low helium massflow | 1,5 kg/s | 1,4 kg/s | 1 s | 1,2 kg/s | 5 s | 1 kg/s | 10 s | - |
| Beam current density | 28uA/cm2 | - | - | 36uA/cm2 | - | - | - | - |

Table 19- limits related to operation with one of two blowers

## Operation at 500 MeV

Operational limits for this operating mode are not yet defined.

# references

- ESS-0051595 AA3 Accident Analysis report loss of Target Wheel cooling during beam on Target

- ESS-0037038 Design Basis for Target Wheel, drive and shaft

- ESS-0049507 Simplified Estimates of limits for pilot TSS

- ESS-0003356 Fatigue assessment of the wheel

- ESS-0034495 Design beam footprints for the Target- Accelerator interface

- ESS-0050081 AA1 Accident Analysis report Target wheel rotation stop during 	    beam on Target wheel

- ESS-0063901 AA2 Accident Analysis report Beam Events

- ESS-003310 Beam on Target requirements

- ESS-0052060 AA5 Accident Analysis report Global bypass and local blocking of target wheel

- ESS-0064357 prompt heat deposition in target wheel

- ESS-0045134 LOCA analysis of the target wheel

- Tomás Mora Aglio- calculation of residual heat in target wheel (pending report)

- Scanscot- Design response spectra for the monolith (pending report)

- ESS-000943 Requirements on spallation material

- ESS-0023776 Target wheel global tolerances and alignment

Document Revision history

| Revision | Reason for and description of change | Author | Date |
| --- | --- | --- | --- |
| 1 | First issue | K.Sjögreen | 2016-06-08 |