Technical report Of the mother board plus daughter board Used like detector and conditioning boards

Abstract

This document is a short technical report of the measurements done on mother board plus daughter board

Document change record

| Issue | Date | Modified Section/Sheet | Comment |
| --- | --- | --- | --- |
| 1 | 21/03/2017 |  | creation |

Written by: Bruno Lagoguez

Table of Contents

1.	Presentation/specification	4

2.	consumption	4

3.	Vout vs Pin	5

4.	rise and fall time	7

5.	Noise	8

6.	Conclusion	8

# Presentation/specification

Measurements has been done with the daughter board equiped with LTC5530 cascaded with mother board.

Figure 1 block diagram of the daughter board

This daughter board can be replace by an other daughter board (not tested and not equipped yet)

Figure 2 Block diagram of the daughter board (analog in)

Figure 3 block diagram of the mother board

## LTC5530

The measurements are done under these conditions with Ra and Rb equal to 1kΩ

# S11 measurement

Measurements to be done (can’t be done now VNA not available) .  And matching to do. Measurements have been done with the matching planned according with the application note.

# consumption

Ic = 43 mA under 24V

This correspond to the supply of the complete board

We have

|  | Power supply | Consumption | Capability |
| --- | --- | --- | --- |
| Primary | MEV1S2405SC 5V | 16mA for the daughter board | 200 mA |
|  | MEA1D2415SC +15V | 8mA for Mother board + 3mA for the daughter board | 33 mA |
|  | MEA1D2415SC -15V | 12 mA for the daughter board | 33mA |
| Secondary | MEA1D2415SC +15V | 8mA | 33mA |
|  | MEA1D2415SC +15V | 8mA | 33mA |

# Vout vs Pin

The signal generator provides the input power to the detector the level is checked with the power meter. At the output noise and voltage is measured with a oscilloscope.

## Ouput of the  LTCC5530

| pin | Vout/2 | slope |  |
| --- | --- | --- | --- |
| 10 | 1.515 |  |  |
| 9 | 1.515 |  | dB/V |
| 8 | 1.51 | 200 | dB/V |
| 7 | 1.255 | 3.921569 | dB/V |
| 6 | 1.01 | 4.081633 | dB/V |
| 5 | 0.79 | 4.545455 | dB/V |
| 4 | 0.6 | 5.263158 | dB/V |
| 3 | 0.44 | 6.25 | dB/V |
| 2 | 0.285 | 6.451613 | dB/V |
| 1 | 0.155 | 7.692308 | dB/V |
| 0 | 0.03 | 8 | dB/V |
| -1 | -0.075 | 9.52381 | dB/V |
| -2 | -0.165 | 11.11111 | dB/V |
| -3 | -0.255 | 11.11111 | dB/V |
| -4 | -0.34 | 11.76471 | dB/V |
| -9 | -0.595 | 19.60784 | dB/V |
| -14 | -0.735 | 35.71429 | dB/V |
| -19 | -0.8 | 76.92308 | dB/V |
| -24 | -0.825 | 200 | dB/V |
| -29 | -0.835 | 500 | dB/V |
| -34 | -0.84 | 1000 | dB/V |
| -39 | -0.84 | #DIV/0! |  |

Vout vs Pin

We recommend using the detector from 7 dBm to -4 dBm, where the slope is stable. The dynamic recommended is 11dB. For example for the forward power, this will give us an ability to measure the power from 2MW to 160 kW (Adding a 26dB attenuator assuming a 60dB coupler).

## Output to the FIM

The attenuator has been tuned to have an amplitude of 1V with +/-0,5V.

# rise and fall time

RF rise and fall time measured with pulsed generator and observed on scope (synchronised with the generator) with probe out of the DUT:

## With LTC5530

Rise time measured for 0dBm input is 1µs.

We did not mount any filter on it.

Fall time is the same and confirms the measurement done on the LTC5530 in standalone.

## Output to the FIM

Figure 4 rise time at the output to the FIM with 7dBm input

## Output of the AD8250

Figure 5 On  channel 1 output of the AD8250 with a gain of 10

The delay is the same than at the output of the detector.

## Output of the isolator for monitoring Iso124 (without filter)

Figure 6 delay at the output for monitoring

The delay becomes 14µs.

## Output of the isolator for monitoring Iso124 (with filter)

A filter has been inserted to remove the DAC effect: R = 200 Ohm in series with C = 18 nH.

Figure 7 delay at the output for monitoring

The delay becomes 30µs.

# Noise

Figure 8 noise measurements

Measurements done at 7dBm input.

Output of the monitoring

74mV PP (trace2) with a slope of 12,75-1,87=1,13 dB/V

So a noise of 0,084 dB

Output of the AD8250:

56mV PP (trace1) with a slope of 12,75-1,87=1,13 dB/V

So a noise of 0,06 dB

Output to the FIM

3mV PP (trace3) with a slope of 1275-188=0,0115 dB/mV

So a noise of 0,034 dB

These results can only be achieved with coaxial cables, excellent ground plane and decoupling and good ground report for the probes.

# Setup

Signal generator keysight MXG N5182B

Power meter Agilent N1912A

Spectrum analyser Keysight MXA N9020B:

Osciloscope R&S Hameg HMO2024

# Conclusion

The board has been measures the next step is to have the board with 10 channels from Atomki, and confirm these results.

# annex

Schematic:

Mother board