ESS Procedure for Wearing and working in Cleanroom

|  | Name | Role/Title |
| --- | --- | --- |
| Owner | Christophe Jarrige | Vacuum system technician |
| Reviewer |  |  |
| Approver |  |  |

| Table of content | Page |
| --- | --- |

1.	Definition	4

2.	Prohibition before entering	5

3.	DRESS PROCEDURE	6

4.	Behaviour in the cleanroom	10

5.	references	10

6.	Document Revision history	10

# Definition

A cleanroom is an environment, in our case used in scientific research, which has a low level of environmental pollutants, such as dust, airborne microbes, aerosol particles and chemical vapours. To work in one, we need to take the appropriate steps to avoid introducing of contamination.

# Prohibition before entering

- Not wear cosmetics, hairspray and perfumes

- Don’t smoke within 45 min of entering in the cleanroom

- Discard gum or anything else in your mouth

- Outdoor shoes must stay outside of cleanroom

- Personnel with colds or severe sunburns

# DRESS PROCEDURE

The rule to put on the cleanroom gear in the correct order is from Top to Bottom

Step 1:   Start putting on under gloves

Step 2: Put on Hair net

Step 3: Put on Face Masks

Step 4: Wear Suit

Don’t let the sleeves touch the ground

Step 5: Pair of shoes

Step 6: get through the curtains

Step 7:  Put on nitrile gloves

Be sure to change nitrile gloves before starting a new activity

# Behaviour in the cleanroom

Walking rapidly or moving unnecessarily is prohibited

All tools or pieces that rest on the surface of table must be placed on a 	cleanroom wipe.

Before and after using, tools must be cleaned with an IPA wipe.

Any work or tools dropped on the floor shall be considered contaminated and 	must be cleaned.

Changes nitrile gloves before starting a new activity

All wastes should be taken out from cleanroom at the end of activities

# references

- ESS vacuum hand book ESS 012894

# Document Revision history

| Revision | Reason for and description of change | Author | Date |
| --- | --- | --- | --- |
| 1 | Adapted to a new template | Christophe Jarrige | 17-11-05 |
|  | <<Keep only full number revisions when approving document>> |  |  |